# Simrad HALO-6 Pulse Compression Radar

Technical Maintenance and Service Log

# Equipment and Service Information

| Equipment Model: | Simrad HALO-6 Pulse Compression Radar (w/ 6-foot open array) |
| --- | --- |
| Vessel Name / ID: |  |
| Scanner Serial Number: |  |
| RI-12 Interface Box S/N: |  |
| Service Date: | 2026-01-26 |
| Technician Name: |  |
| Technician Signature: |  |

# Preventive Maintenance Checklist

Perform these checks periodically to ensure optimal performance and longevity of the radar system. Status options: Pass, Fail, N/A (Not Applicable).

| Task # | Category | Task Description | Status | Comments |
| --- | --- | --- | --- | --- |
| A. Physical & Mechanical Inspection |  |  |  |  |
| A.1 | Antenna Array | Visually inspect the 6-foot open array antenna for cracks, physical damage, or delamination. Ensure it is clean and free of debris. |  |  |
| A.2 | Antenna Cleaning | Clean the antenna array using a soft cloth, fresh water, and a mild, non-abrasive soap. Rinse thoroughly. Do not use chemical solvents or high-pressure washers. |  |  |
| A.3 | Pedestal Housing | Inspect the pedestal housing for cracks, seal integrity, and signs of water ingress. Check that all covers are secure. |  |  |
| A.4 | Mounting Hardware | Check all mounting bolts for the pedestal for proper tightness and signs of corrosion. Ensure marine-grade stainless steel hardware is used. |  |  |
| A.5 | Antenna Rotation | With the system powered down, manually check that the antenna can swing freely without obstruction or excessive resistance. |  |  |
| B. Electrical & Network Inspection |  |  |  |  |
| B.1 | Power Cabling | Inspect the main power cable to the RI-12 Interface Box. Check for chafing, cuts, UV damage, and corrosion at the connection points. Ensure connections are tight. |  |  |
| B.2 | Interconnect Cable | Inspect the cable between the RI-12 Interface Box and the scanner pedestal. Check for damage to the cable and connectors (RJ45). Ensure a secure connection. |  |  |
| B.3 | Ethernet Network | Inspect the Ethernet cable connecting the RI-12 to the MFD or network switch (e.g., NEP-2). Check for secure connections and cable integrity. |  |  |
| B.4 | RI-12 Interface Box | Open the RI-12 Interface Box. Visually inspect internal connections for tightness, signs of corrosion, or moisture. Ensure the lid seal is in good condition. |  |  |
| B.5 | Voltage Check | With the system on, use a multimeter to measure the DC voltage at the RI-12 power input terminals. Verify it is within the specified range (typically 12/24V DC, check manual for specifics). |  |  |
| B.6 | NMEA 2000 Network | If connected, inspect the NMEA 2000 backbone. Verify the presence of exactly two terminators, secure T-connectors, and a stable power supply (9-16V DC). |  |  |
| C. System & Software Checks |  |  |  |  |
| C.1 | Firmware Version | Check the radar's installed software version via the MFD's network device list. Compare with the latest version available on the Simrad Yachting support website. |  | Current: ________ Latest: ________ |
| C.2 | System Power-Up | Power on the MFD and radar system. Confirm the radar is detected by the MFD within the normal 30-40 second startup time and does not display a "No Scanner" error. |  |  |
| C.3 | Transmit Test | Place the radar in Transmit (Tx) mode. Confirm the open array antenna begins to rotate smoothly and quietly. Listen for any unusual noises from the pedestal motor. |  |  |
| C.4 | Performance Test | Verify a clear radar picture is painted on the MFD. Check for good target definition at both short (e.g., <1 NM) and long ranges (e.g., >12 NM). |  |  |
| C.5 | Mode Functionality | Briefly test key operational modes such as Harbour, Offshore, Weather, and Bird to confirm they activate and adjust the display as expected. |  |  |
| C.6 | Dual Range Test | Activate Simultaneous Dual Range operation. Confirm the MFD displays two independent radar ranges and that both update correctly. |  |  |

# Corrective Maintenance Log

Reported Fault / Symptom Description:

Troubleshooting Steps & Findings

Follow these guided steps based on the reported symptom. Document all actions and findings.

| Step | Action Taken / Check Performed | Result / Finding | Corrective Action / Notes |
| --- | --- | --- | --- |
| Symptom Group 1: No Power, Intermittent Power, System Not Detected |  |  |  |
| 1.1 | Check dedicated breaker/fuse for marine electronics. Cycle breaker off and on. Inspect fuse condition. |  |  |
| 1.2 | Use a multimeter to confirm voltage at the RI-12 Interface Box power input terminals. |  | Voltage Reading: ______ V |
| 1.3 | Inspect power cable from source to RI-12 for damage. Check for corrosion (blackened copper) at all connection points. |  |  |
| 1.4 | Power cycle the entire system (MFD and radar breaker). Wait 60 seconds before restoring power. |  |  |
| Symptom Group 2: "No Scanner", "Scanner Not Detected", "Invalid Source", "Unknown Radar" |  |  |  |
| 2.1 | On the MFD, navigate to Settings > Network > Device List. Check if the HALO-6 radar is listed. |  | If not listed, indicates a power or network communication failure. |
| 2.2 | Inspect the Ethernet cable from the RI-12 to the MFD/network switch. Reseat both ends. Check for bent pins or corrosion. |  |  |
| 2.3 | Inspect the interconnect cable from the RI-12 to the scanner pedestal. Reseat both ends. Check for damage. |  | This is a common point of failure. |
| 2.4 | If possible, test with a known-good Ethernet cable to isolate a faulty cable. |  |  |
| Symptom Group 3: "No Spoke Data", Poor Image Quality, No Targets |  |  |  |
| 3.1 | Confirm radar is in Transmit (Tx) mode on the MFD. |  |  |
| 3.2 | Observe the scanner pedestal. Confirm the antenna is rotating when in Tx mode. |  | If not rotating, see Symptom Group 4. |
| 3.3 | Check MFD radar settings. Adjust Gain, Sea Clutter, and Rain Clutter. Try switching to a preset mode (e.g., Offshore). |  |  |
| 3.4 | Power cycle the system. A "No Spoke Data" error can indicate a temporary internal scanner error that a restart may resolve. |  | If persistent, may indicate an internal fault requiring service. |
| Symptom Group 4: Antenna Not Rotating |  |  |  |
| 4.1 | Listen for the sound of the motor drive inside the pedestal when in Tx mode. |  | Motor hum indicates power is reaching the motor. |
| 4.2 | Power down system. Visually inspect the antenna path for any physical obstructions. |  |  |
| 4.3 | Check MFD for any motor-related error codes (e.g., Motor drive fault, Motor or antenna has stalled). |  | Refer to Error Code table below. |
| 4.4 | If accessible and technician is qualified, inspect internal drive belt for breakage or slippage. |  | Requires pedestal disassembly. Follow service manual procedures. |

Final Diagnosis & Resolution Summary:

# Common Error Codes & Resolutions (Reference)

This list is based on Simrad HALO series documentation. Specific codes may vary. Always confirm with the latest service manual.

| Error Code | Description | Recommended Action |
| --- | --- | --- |
| 0x0001000C | Scanner not detected | 1. Check interconnect cable connections. 2. Power cycle the radar system. 3. Check input voltage to RI-12. |
| - | No spoke data | 1. Check supply voltage/current. 2. Power cycle the system. 3. Check scanner cable/RJ45. 4. If persistent, indicates possible internal fault; contact service. |
| 0x00010018 | Radar interface box fault | Check the interconnection cable between the scanner and RI-12 for damage. Check RI-12 power. |
| 0x00010019 | Low battery voltage | 1. Check and recharge vessel batteries. 2. Verify integrity of power supply wiring. 3. Restart the radar. |
| 0x00010003 | Motor drive fault | Indicates an internal fault with the motor control system. Power cycle system. If persistent, contact service. |
| 0x0001001A | Motor or antenna has stalled | Power down. Check for physical obstruction preventing antenna rotation. If clear, may indicate mechanical or motor fault. Contact service. |
| 0x00010001 | Zero bearing sensor fault | Internal sensor failure. Contact service. |
| 0x00000001 | Radar saved settings corrupted | Radar will revert to factory defaults. Re-enter user and installation settings. |

# Final System Test & Verification

To be completed after any maintenance or corrective action.

| Test # | Verification Check | Pass / Fail | Comments |
| --- | --- | --- | --- |
| 1 | System powers on correctly and radar is detected by MFD without errors. |  |  |
| 2 | Radar transmits and antenna rotates smoothly. |  |  |
| 3 | Clear radar image is displayed with good target separation. |  |  |
| 4 | All operational modes (Harbour, Offshore, etc.) and Dual Range function correctly. |  |  |
| 5 | System operates for a minimum of 15 minutes without recurrence of the original fault. |  |  |